Mailam Engineering College

Mailam (Po), Villupuram (Dt). Pin: 604 304

##### (Approved by AICTE, New Delhi, Affiliated to Anna University, Chennai

##### & accredited by Tata consultancy services)

##### Department Of Electronics and Communication Engineering

Sub Code/Name		: EC2402/DIGITAL SIGNAL PROCESSING

Unit No				: IV

Year/Sec 			         : III/A& B &C

Total No of Pages & Copies	:       /

UNIT IV		FINITE WORD LENGTH EFFECTS

Fixed point and floating point number representations – Comparison – Truncation andRounding errors - Quantization noise – derivation for quantization noise power –coefficient quantization error – Product quantization error - Overflow error – Roundoffnoise power - limit cycle oscillations due to product roundoff and overflow errors – signalscaling

Updated Questions: [EC2302(ECE),CS2403(CES,IT),EC2314(EEE) NOV/DEC 2013]

Part-A

- Q.No. 2		:Page No.2 (Text book 3)

- Q.No. 4,5	 	:Page No.2,3 (Text book 3)

- Q.No. 7	 	:Page No.3 (Text book 3)

- Q.No. 21	 	:Page No.6 (Text book 3)

- Q.No. 23	 	:Page No.6 (Text book 3)

- Q.No. 32 		:Page No.9 (Text book 3)

Part-B Theory

- Q.No. 1 		:Page No.10 (Text book 3)

- Q.No. 2		:Page No.13 (Text book 3)

- Q.No. 3 		:Page No.14 (Text book 3)

- Q.No. 4 		:Page No.15 (Text book 3)

- Q.No. 5 		:Page No.18 (Text book 3)

Part-B Problems

- Q.No. 1 		:Page No.18 (Text book 3)

- Q.No. 6 		:Page No.23 (Text book 3)

- Q.No. 7 		:Page No.24 (Text book 3)

- Q.No. 10 		:Page No.28 (Text book 3)

Referred Text Book:

- S.Salivahanan, A. Vallavaraj, C. Gnanapriya, Digital Signal Processing, Tata McGraw Hill International, 2007

- Nagoor kani, Digital Signal Processing, , second edition Tata McGraw Hill International, 2012

- Ramesh babu, Digital Signal Processing, Lakshmi publications (P) LTD.

Prepared by

- Mrs.R.Gayathri, Asst.Prof./ECE

- Mr.C.V.Venkatasamy, Asst.Prof./ECE

- Mr.K.Balaji, Asst.Prof./ECE

Verified by

Year Co-ordinator			HoD			          Principal

- What is a finite word length effect?

- DSP algorithms are realized with special purpose digital hardware or as programs. In both the cases the numbers and co-efficient are stored in finite length registers.

- Therefore the co-efficient and number are quantized by truncating or rounding when they are stored. This creates error in the output.

- These type of effect due to finite precision representation of numbers in digital system are called finite word length effects.

- What are the errors arise due to quantization of numbers?

- Input quantization error.

- Product quantization error.

- Co-efficient quantization error.

- What is fixed point representation?  (Or) How the numbers are represented in Fixed point  arithmetic and what are its different types?(May-2011)

- Position of the binary point is fixed for fixed point representation.

Positive numbers:

- Bit to the right -> fractional part of the number.

- Bit to the  left   -> integer part of the number.

Example:  binary number -> 01.1100

Decimal value->1 . 75

Left   right

Integer    fractional part

Negative number:

- Three different formats for representing  negative numbers in fixed point arithmetic.

- Sign-magnitude form.

- One’s – complement form.

- Two’s – complement form.

- What is floating point arithmetic?  (Or) How the numbers represented in floating poing arithmetic?(Nov-2011)

In floating point representation, the binary point can be shifted to desired position so that bits in the integer part and fraction part of a number can be varied.

Positive number:

F=2cM

M-> Mantissa, is a fraction 12≤M≤1

c-> exponent (+ve, or –ve )

Negative number:

- Mantissa -> fixed point number.

- Sign of the number is obtained from the first bit of mantissa.

Example:

| Decimal | floating point | Binary form | Exponent | Mantissa |
| --- | --- | --- | --- | --- |
| 4.5 | 23X0.5625 | 2011X0.1001 | 011 | 01001 |
| 1.5 | 21X0.75 | 2001X0.1100 | 001 | 01100 |
| 0.625 | 20X0.625 | 2000X0.1010 | 000 | 01010 |

Sign Bit

- What is meant by truncation? (Nov-2010)

Truncation is the process of reducing the size of binary number by discarding all bits less significant than least significant bit that is retained.

Example: Truncate the binary number from 8 bits to 4 bits.

0.01011000 0.0101 (8 bits) (4-bits)

1.101001111.1010 (8 bits) (4-bits)

Note: In truncation of a binary number to b bits all the less significant bits beyond bth bit are discarded

- What is meant by rounding?

- Rounding is the process of reducing the size of a binary number to ‘b’ bits of finite word size, such that the rounded b-bit number is closed to the original unquantized number.

- The rounding process consists of truncation and addition .

- In rounding of a number to b bits, first the unquantized number is truncated to b bits by retaining the most significant b bits. Then zero or one is added to LSB of the truncated number depending on the bit that is next to LSB.

- If the bit next to LSB is zero ,then zero is added to the LSB of the truncated number.

- If the bit next to LSB is one, then one is added to LSB of the truncated number.

Example:

Round off the binary number to 3 bits

0.11010 is rounded to 3 bits by following steps.

We know that round off method is done by two steps.

- First, truncate the given number into 3 bits. So

0.11010    truncate to 3 bits                             0.110

- Second, Addition is done. Here for given number (0.11010), the bit next to LSB i.e. fourth bit is 1. So add 1 with the truncated number in first step. i.e.

- 0.110

+

1

0.111

- Rounding up or down will have negligible effect on accuracy of computation.

- Compare fixed point and floating point representation?  (Nov 2012 & May 2012)

| Fixed point representation | Floating point representation |
| --- | --- |
| - Positive of binary point is fixed. | Position of the binary point is variable. |
| - Resolution is uniform | Resolution is variable |
| - In a b-bit, the range of number represented is less when compared to floating point representation. | In a b-bit, the range of number represented is large when compared to fixed point representation. |
| - Fast operation | slow operation |
| - Small dynamic range | increased dynamic range |
| - Overflow occurs in addition | overflow does not arise |
| Used in small computers | Used in larger, general purpose computers. |
| round off errors can occur with both addition and multiplication | Round off errors occurs only in Addition |

- What is product quantization error?(Nov 2010)

- In digital computations, output of multiplier or the products are quantized to finite word length in order to store then in register and to be used in subsequent calculations.

- The error due to the quantization of the output of multiplier is referred to as product quantized error.

- Multiplication of a b bit data with a b bit coefficient results a product having 2b bits

- What are the effects of finite word length?(may2009)

- Errors due to quantization of input data.

- Errors due to quantization of Filter coefficients

- Errors due to Rounding the product in multiplications

- Limit cycles due to product quantization and overflow in addition.

- What are the types of number representation?

- Fixed point number representation

- Floating point number representation

- Block floating point representation

- What is input quantization error?(Nov 2009,2013)

The conversion of a continuous time input signal into digital value produces an error, which is known as input quantization error. This error arises due to the representation of the input signal by a fixed number of digits in ADC conversion process.

- What is coefficient quantization error or coefficient inaccuracy?(Nov 2011)

What is importance of poles in filter design ? (CSE-Nov 2011)

- The filter coefficients are computed to infinite precision theory. But in digital computation the filter coefficients are represented in binary and are stored in registers. If a b bit register is used, the filter coefficients must be rounded or truncated to b bits , which produce an error. It known as coefficient quantization error or quantization of filter coefficients.

Effects of coefficient quantization error:

- The location of poles and zeros of the digital filters directly depends upon the value of filter coefficients.

- The coefficient quantization will modify the value of poles and zeros and so the location of poles and zeros will be shifted from the desired position. This will create deviation in the frequency response of the filter.

- What are the three different form of fixed point arithmetic?

Sign-magnitude form , one’s complement form and two’s complement form.

- What is quantization noise? (Or) ADC conversion noise ?(nov 2010)

The difference signal e(n) = xq(n) – x(n) is called quantization noise or ADC conversion noise.

Where, e(n) = error signal

Xq(n)= quantized signal

X(n)=actual signal

- What are the common methods of quantization?

Truncation and rounding.

- What is zero input limit cycle oscillations? (or) What is limit cycle oscillation ? (May2010,2011,2012,Nov-2011) (CSE-May 2011,Nov 2012)

In some systems, when the input is zero or some non zero constant value the nonlinearities due to the finite precision arithmetic operations often cause periodic oscillations to occur in the output. Such oscillations in recursive systems are called limit cycle oscillations. These oscillations will continue to remain in limit cycle even when the input is made zero. Hence, these limit cycle are also called zero input limit cycles.

- What is dead band?(May2008)(May 2012)

The limit cycles occur as a result of the quantization effects in multiplications. The amplitude of the output during a limit cycle are confined to a range of values that is called the dead band of the filter.

The dead band is given by

Dead band=±2-b1-a=-2-b1-a,2-b1-a

- What is overflow limit cycle oscillation and how it is eliminated? (May 2009)

State the methods to prevent overflow(Nov-2013)

- In fixed point addition the overflow occurs when the sum exceeds the finite word length of the register used to store the sum. The overflow in addition may lead to oscillations in the output .

- It is caused by binary arithmetic which makes the filter output oscillates between maximum and minimum amplitudes. Such limit cycles have been referred to as overflow limit cycle oscillation.

Elimination of  overflow limit cycle oscillation

- The overflow limit cycles can be eliminated by using saturation arithmetic or by scaling the input signal to the adder.

- What is signal scaling?(May 2011 & 2012)

Due to overflow, the saturation arithmetic eliminates limit cycles but it causes undesirable signal distortion due to the nonlinearity of the clipper. In order to limit the amount of non linear distortion, it is important to scale the input signal and the unit sample response between the input and any internal summing node.

- Why rounding is preferred to truncation in realizing digital filter.(May2008)

The mean of rounding error is zero and the variance of the rounding error signal is low. So the rounding is preferred than the truncation.

- Explain briefly the need for scaling in the digital filter .(May 2011)

To prevent overflow, the signal level at certain points in the digital filters must be scaled so that no overflow occurs in the adder.

- What are the advantages of floating point arithmetic?(May2012)

Larger dynamic range.

Overflow in floating point representation is unlikely.

- Which realization is less sensitive to the process of quantization?

Cascade form

- What are the two kinds of limit cycle behavior in DSP?

Zero input limit cycle oscillations

Overflow limit cycle oscillations

- What is meant by quantization step size?(Nov2012)

Let us assume a sinusoidal signal varying between +1 and -1 having a dynamic range 2. If ADC used to convert the sinusoidal signal employs b+1 bits including the sign bit, the number levels available for quantizing x(n) is 2b+1.

q= R/2b

Where R is dynamic range. Assume R=2 then,

q= 2/2b+1   =2-b

Part-B

- Explain the detail the 3 types of quantization error that occur due to the finite word length of register.

- DSP algorithms are realized with special purpose digital hardware or as programs. In both the cases the numbers and co-efficient are stored in finite length registers.

- Therefore the co-efficient and number are quantized by truncating or rounding when they are stored. This creates error in the output.

- These type of effect due to finite precision representation of numbers in digital system are called finite word length effects.

- The following errors arises due to quantization of numbers.

- Input quantization error.

- Product quantization error.

- Co-efficient quantization error.

Input quantization error:

The conversion of a continuous time input signal into digital value produces an error, which is known as input quantization error. This error arises due to the representation of the input signal by a fixed number of digits in A/D conversion process.

Product quantization error:

Product quantization error arise at the output of a multiplier. Multiplication of a b-bit data with a b-bit coefficient results a product having 2b bits.

Since a b-bit register is used, the multiplier output must be rounded or truncated to b-bits which produced an error.

Coefficient quantization error:

The filter coefficients are computed to infinite precision in theory. If they are quantized, the frequency response of the resulting filter may differ from the desired response.. If the poles of the desired filter are close to the unit circle, then those of the filter with quantized coefficient may we outside the unit circle leading to instability.

The other errors arising from quantization are round-off noise and limit cycle oscillations.

- The output of an A/D converter is applied to a digital filter with the system function, Hz=0.5zz-0.5 find the output  noise power from the digital filter when the input signal is quantized to have

8 bits.

Given data:

Hz=0.5zz-0.5

Find the output noise power from the digital filters, when the input signal is quantized to have 8-bit.

Solution:

The input quantization noise power (σe2) is given by,

σe2=2-2b12

Given b=8, therefore, σe2=2-2812=1.27X10-6

The output noise power is given by,

σe02=σe22πjHz Hz-1 z-1dz

σe02=σe22πj0.5zz-0.50.5z-1 z-1-0.5 z-1dz

σe02=σe22πj0.25 z z-1(z-0.5)(1-0.5z) dz

σe02=σe22πj0.25(z-0.5)(1-0.5z) dz

The above integral can be evaluated by the method of residues.

I=sum of residues at the poles within the unit circle within z<1

The poles are z=0.5 (inside the unit circle, so it is stable pole.)

z=10.5=2 outside the unit circle,so it is unstable pole.

NOTE: Only consider the stable poles.i.e. Poles lie inside the unit circle.

I=z-0.50.25z-0.51-0.5z   | z=0.5

I=0.333

The output noise power (σe02)is given by,

σe02=σe2 . I

σe02=0.333σe2

=0.333(1.27X 10-6)

σe02=0.423X10-6

Quantization of Filter Coefficients

In the design of a digital filter the coefficients are evaluated with infinite precision. But they are limited by the word length of the register used to store the coefficients. Usually the filter coefficients are quantized to the word size of the register used to store them either by truncation or by rounding. The location of poles and zeros of the digital filters directly depends on the value of filter coefficients. The quantization of the filter coefficients will modify the value of poles and zeros and so the location of the poles and zeros will be shifted from the desired location. This will create deviation in the frequency response of the System. Hence we obtain a filter having a frequency response that is different from the frequency response of the filter with unquantized coefficients. The sensitivity of the filter frequency response characteristics to quantization of the filter coefficients is minimized by realizing the filter having a large number of poles and zeros as an Interconnection of second-order section. Therefore, the coefficient quantization has less effect in cascade realization when compared to other realizations.

- For a second order IIR filter  Hz=11-0.9 z-1(1-0.8z-1) find the effect of shift in pole location with 3-bit coefficient presentation in direct from and cascade form.

Given data:

Hz=11-0.9 z-1(1-0.8z-1)

Hz=z2z-0.9 (z-0.8)

The roots of the denominator of H(z) are the original poles

P1=0.9, P2=0.8

Direct form:

Hz=11-0.9 z-1(1-0.8z-1)

Hz=11-0.8z-1-0.9 z-1+0.72 z-2

Hz=11-1.7z-1+0.72 z-2

Let us quantize the coefficient by truncation of 3 bit 1.7 convertbinary1.10112convert 3 bits1.1012convert decimal2.62510

1.101=1X21       .    1X2-1+0X2-2 + 1X2-3

0.70X2=1.4					      =2           .     0.5+      0    + 0.125

0.40X2=0.8					    =2.62510

0.80X2=1.6

0.60X2=1.2

0.72 convertbinary0.10112truncate 3 bits0.1012convert decimal0.6210

0.72X2=1.44				0.1012  =1X2-1+0X2-2+1X2-3

0.44X2=0.88				                   =0.5      +  0   + 0.125

0.88X2=1.76				                    =0.62510

0.76X2=1.52

Let H(z) be the transfer function. After quantizing the co-efficient.

H(z)= 11-2.625 z-1+0.625z-2

The new poles are,

Pd1=2.360

Pd2=0.264

Compare P1, Pd1 and P2, Pd2. We can observe that there is a lot of difference in the position of quantize and unquantized poles.

Cascade form:

We know that for cascade form,

Hz=H1z. H2(z)

Given,

Hz=11-0.9 z-1(1-0.8z-1)

Therefore, H1z=11-0.9 z-1,  H2z=11-0.8 z-1

P1=0.9, P2=0.8

Let us quantize the co-efficient of H1zand H2z

0.9 convertbinary0.11102truncate 3 bits0.1112convert decimal0.87510

0.9X2=1.80					0.11121X2-1+1X2-2+1X2-3

0.80X2=1.60						=0.5+0.25+0.125

0.60X2=1.20						=0.875

0.20X2=0.40

0.8 convertbinary0.11002truncate 3 bits0.1102convert decimal0.7510

0.80X2=1.60				0.11021X2-1+1X2-2+0X2-3

0.60X2=1.20						=0.5+0.25

0.20X2=0.40						=0.75

0.40X2=0.80

H1(z)= 11-0.875 z-1=PC1=0.875

H2(z)= 11-0.75 z-1=PC2=0.75

On comparing the poles of cascade system after quantization with the unquantized coefficients P1, Pc1 and P2, Pc2 are having slight difference in their poles.

Conclusion:

• From direct form, we can see that the quantized poles deviate very much from the original poles .

• From cascade form, we can see that one pole is exactly the same while the other pole is very close to the original pole.

- For the second order IlR filter, the system function is,

H(Z)=11-0.5z-1 (1-0.45z-1)

Find  the effect of shift in pole location with 3 bit coefficient representation in direct and cascade forms.(MAY- 2012).

Solution:

HZ=11-0.5z-1 1-0.45z-1=z2z-0.5(z-0.45)

Original poles of  H(Z)  is P1= 0.5 and P2= 0.45.

CASE 1: DIRECT FORM

HZ=11-0.5z-1 1-0.45z-1=1(1-0.95z-1+0.225z-2)

Quantization of coefficient by truncation

convert to binary		       truncate to 3 bits  	                    convert to decimal

0.9510                                                  0.11112                                 0.1112                                        0.87510

convert to binary		        truncate to 3 bits  	                     convert to decimal

0.22510                                                  0.00112                                 0.0012                                        0.12510

HZ=1(1-0.875z-1+0.125z-2)

H(Z)=11-0.695z-1 (1-0.179z-1)

The poles are at P1 = 0.695 and P2 = 0.179.

Case (ii) Cascade Form

Given, 
H(Z)=11-0.5z-1 (1-0.45z-1)

Quantization of coefficient by truncation

convert to binary		       truncate to 3 bits  	                    convert to decimal

0.510                                                  0.10002                                 0.1002                                        0.510

convert to binary		        truncate to 3 bits  	                     convert to decimal

0.4510                                                  0.01112                                 0.0112                                        0.37510

H(Z)=11-0.5z-1 (1-0.375z-1)

The poles are P1 = 0.5  and P2 = 0.375

Conclusion:

• From direct form, we can see that the quantized poles deviate very much from the original poles .

• From cascade form, we can see that one pole is exactly the same while the other pole is very close to the original pole.

Product Quantization Error

In fixed point arithmetic the product of two b bit numbers results in number of 2b bits length. If the word length of the register used to store the result is b bit, then it is necessary to quantize the product to b bits, which produce an error known as product quantization error or product round off noise. In realization structures of digital system, multipliers are used to multiply the signal by constants.

The model for fixed point round off noise following a multiplication is shown in Figure . The multiplication is modeled as an infinite precision multipliers followed by an adder where round off noise is added to the product so that overall result equals some quantization level. The roundoff noise sample is a zero mean random variable with a variance (2-2b/3), where b is the number of bits used to represent the variables.

In general the following assumptions are made regarding the statistical independence of the various noise sources in the digital filter.

I. Any two different samples from the same noise source are uncorrelatcd.

2. Any two different noise source, when considered as random processes are uncorrelated.

3. Each noise source is uncorrelated with the input sequence.

Let ek(n) be the error signal from kth noise source, hk(n) the impulse response for kth noise source and Tk(n) the noise transfer function (NTF) for kth noise source.

Variance of kth noise source σ2ek = q212=2-2b3(Assume R=2,w.k.t.q=R2b )

PROBLEM 1

In the IIR system given below the products are rounded to 4 bits (including sign bits). The system function is

H(Z)=11-0.35z-1 (1-0.62z-1)

Find the output roundoff noise power in (a) direct form realization and (b) cascade form realization.

Solution

- Direct Form Realization

H(Z)=11-0.35z-1 (1-0.62z-1)

HZ=1(1-0.97z-1+0.217z-2)

Direct form realization of H (z) is shown in Figure

The variance of the error signal is,

Here R is not given. So take R = 2 V and b = 4 bits

q=R2b=224=123=18

σe2=(18)212=q212

σe2=1.3021X 10-3

Output noise power due to the noise signal el (n) is,

σe012= σe22πjHz Hz-1 z-1dz

Here,

H(Z)=11-0.35z-1 (1-0.62z-1)

H(Z)=z2z-0.35(z-0.62)

Therefore,

σe012=σe22πjz2z-0.35(z-0.62)z-2 (z-1-0.35) (z-1-0.62) z-1dz

σe012 =σe22πjz-1z-0.35(z-0.62) (z-1-0.35) (z-1-0.62)dz

The stable poles of H(z) are PI = 0.35 and P2= 0.62 and unstable poles of H(z) are P3 = 2.86 and P4 = 1.62. For taking residue only consider the stable poles.

ResHzHz-1z-1z=0.35=

=z-0.35z-1z-0.35(z-0.62) (z-1-0.35) (z-1-0.62)z=0.35

= -1.8867.

ResHzHz-1z-1z=0.62=

=z-0.62z-1z-0.35(z-0.62) (z-1-0.35) (z-1-0.62)z=0.62

=4.7640.

Total = ResHzHz-1z-1z=0.35+ ResHzHz-1z-1z=0.62

= -1.8867 +  4.7640.

= 2.8773.

Therefore,

σe012= σe22πjHz Hz-1 z-1dz

= 1.3021 X 10-3 X 2.8733

= 3.7465 X 10-3

Here the output noise due to error source e2(n) is same as that of e1 (n), i.e.,

e2 (n) noise power = noise power of e1(n)

σe012=σe022

= 3.7465 x 10-3

Total output noise power due to all the noise sources is,

σe02=σe01+2σe022

σe02=7.493 X 10-3

(b) Cascade Realization

Given

H(Z)=11-0.35z-1 (1-0.62z-1)

Let H(z) =H1(Z)H2(Z), i.e.,

H1z=11-0.35z-1and H2z=11-0.62z-1

Case (i)     H(z) = H1(Z)H2(Z)

The cascade form realization of H(z) is shown in Figure

The order of cascading is H1(Z)H2(Z). Output noise power due to errorsignal e1(n) is

σe012= σe22πjHz Hz-1 z-1dz

= 3.7465 X 10-3[refer direct form ]

Output noise power due to the error, signal e2(n) is

σe022= σe22πjH2zH2z-1z-1dz

H2zH2z-1z-1 =  z-1z-0.62(z-1-0.62)

ResH2zH2z-1z-1z=0.62=z-0.62z-1(z-0.62) (z-1-0.62)z=0.62

=1.6244

σe022= σe22πjH2zH2z-1z-1dz

=1.3021 X 10-3 X 1.6244

= 2.1151 X 10-3

Total  Output noise power

σe02=σe01+2σe022

= 3.7465 x 10-3+2.1151 x 10-3

σe02=5.8616 X 10-3

Case (ii) The order of cascading is H(z) = H2(Z)H1(z) and is shown in Figure

The output noise power due to error source e1is,

σe012=3.7465 X 10-3

The output noise power due to error source e1(n) is,

σe022= σe22πjH1zH1z-1z-1dz

H1zH1z-1z-1 =  z-1z-0.35(z-1-0.35)

ResH1zH1z-1z-1z=0.35=z-0.35z-1(z-0.35) (z-1-0.35)z=0.35

= 1.1396

σe022=1.1396 X 1.3021 X 10-3

= 1.4839 X 10-3

Total output noise power

σe02=σe01+2σe022

=3.7465  10-3 + 1.4839 X 10-3

σe02=5.2304 X 10-3

Conclusion: Thus, in cascade form realization, the product noise round off power is less in case (ii) when compared to case (i) and also direct form realization.

Limit Cycles in Recursive System

1.Zero-Input Limit Cycles

In recursive systems, when the input is zero or some non-zero constant value, the non-linearity’s due to finite precision arithmetic operation may cause periodic oscillations, in the output. During periodic oscillations, the output y(n) of a system will oscillate between a finite positive and negative value for increasing n or the output will become constant for increasing n. Such oscillations are called limit cycles. If the system output enters a limit cycle, it will continue to remain in limit cycle even when the input is made zero. Hence, these limit cycles are also called zero input limit cycles.

Consider the following difference equation of first order system with one pole only.

y(n) = ay(n - 1) +x(n)

The system has one product ay(n - 1). If the product is quantized to finite word length then the response y(n) will deviate from actual value. Let y' (n) be theresponse of the system when the product is quantized.

y'(n) = Q [ay'(n - 1)] +x(n)

Let y'(n) = 0, for n <0 and a =1/2

xn=0.875,  &n=00,  &n≠0

Let the product be quantized to three bit (excluding sign bit) binary by rounding

when n=0,

y’(n)=Q[ay’(n-1)+ x(n)]

Y (0) = Q [1/2 Y (-1) +x(O)]

= Q [1/2 x 0] + 0.875 = 0.87510 = 7/8

When n = 1,

y'(1) = Q[ay'(O)] +x(l)

= Q [1/2  x 0.875] + 0

= Q[1/2 * 7/8] = Q[7/16]=Q[0.4375]

Covert to binary                                         add sign bit 			round to 3 bit

0.437510                                                  .0111002                                      0.0111002                                0.1002

extract sign bit 	                 convert to decimal

0.1002                                            0.510

Y(1) = 0.510

When n=2,

y'(2) = Q[ay'(1)] +x(2)

= Q [1/2  x 0.5] + 0

= Q[1/4] = Q[.25]

Covert to binary                               round to 3 bit                   convert to decimal

0.2510                                                  0.010002                                  0.0102                                                        0.2510

Y(2) = 0.2510

When n=3,

y'(3) = Q[ay'(2)] +x(3)

= Q [1/2  x 0.25] + 0

= Q[1/8] = Q[.125]

Covert to binary                               round to 3 bit                   convert to decimal

0.12510                                                  0.0010002                                  0.0012                                                      0.12510

Y(3) = 0.12510

When n=4,

y'(4) = Q[ay'(3)] +x(4)

= Q [1/2  x 0.125] + 0

= Q[1/16] = Q[.0625]

Covert to binary                               round to 3 bit                   convert to decimal

0.062510                                        0.00010002                                  0.0012                                    0.12510 = 1/8

Y(4) = 0.12510

For all values of n , where n ≥ 3, the y'(n) = 1/8 = 0.001. Hence, the output becomes constant for n ≥3. Also for n≥3, the input x(n) is zero. Therefore, the system enters a limit cycle even though the input becomes zero for      n ≥3.

Dead Band

In a limit cycle the amplitudes of the output are confined to a range of values, which is called the dead band of the filter.

For a first order system described by the equation, y(n) = ay(n - 1) + x(n), the dead band is given by

The limit cycles occur as a result of the quantization effects in multiplications. The amplitude of the output during a limit cycle are confined to a range of values that is called the dead band of the filter.

The dead band is given by

Dead band=±2-b1-a=-2-b1-a,2-b1-a

where b=number of bits (including sign bits) used to represent the product. Fora second order system described by the difference equation

y(n) = a1y(n - 1) +a2y(n - 2) +x(n), the dead band is

Dead band=±2-b1-a2=-2-b1-a2,2-b1-a2

Problem1

Explain the characteristics of a limit cycle oscillation with respect to the system described by the equation

y(n) = 0.95y(n - 1) + x(n).

Determine the dead band of the filter.(Assume sign magnitude is 5 bit).(May-2012)

Given that

y(n) = 0.95y(n - 1) + x(n)

Let y'(n) be the response of system when the product is quantized by rounding .

.'. y'(n) = Q[0.95y(n - 1)] +x(n)

where Q is quantization. Given that 5 bit sign-magnitude binary representation with 4 bit for magnitude and 1 bit for sign.

Let

y' (n) = 0 for n <0

and

xn=0.75,  &n=00,  &n≠0

when n=0,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (0) = Q [0.95 Y (-1) +x(O)]

= Q [0.95 x 0] + 0.75

= 0.75

Y’ (0) = 0.7510 = 0.11002

Covert to binary                      round to 4 bit                   convert to decimal

0.7510                                        0.110002                                  0.11002                                    0.7510

when n=1,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (1) = Q [0.95 Y (0) +x(1)]

= Q [0.95 x 0.75] + 0

= Q[0.7125]

Covert to binary                      round to 4 bit                   convert to decimal

0.712510                                        0.101102                                  0.10112                                    0.687510

Y(1) = 0.687510

when n=2,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (2) = Q [0.95 Y (1) +x(2)]

= Q [0.95 x 0.6875] + 0

= Q[0.653125]

Covert to binary                      round to 4 bit                   convert to decimal

0.65312510                               0.101002                                  0.10102                                    0.62510

Y(2) = 0.62510

when n=3,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (3) = Q [0.95 Y (2) +x(3)]

= Q [0.95 x 0.625] + 0

= Q[0.59375]

Covert to binary                      round to 4 bit                   convert to decimal

0.59375510                               0.100112                                  0.10102                                    0.62510

Y(3) = 0.62510

Thus, y' (2) =y' (3), and hence for all values of n ≥ 2, y' (n) will remain as 0.625.

Therefore, the system enters into the limit cycle when n = 2.

For the first order system with only one pole, dead band is given by

Dead band=±2-b1-a=-2-b1-a,2-b1-a

where b is number of bit in binary representation and lal= 0.95

Dead band=±2-51-0.95=-2-51-0.95,2-51-0.95

=[ -0.625 , + 0.625]

Problem 2

An IIR causal filter has the system function

Hz=zz-0.97

Assume that the input signal is zero valued and the computed output signal values are rounded to one decimal place. Show that under those stated conditions, the filter output exhibits dead band effect. What is the dead band range? (Anna University, May, 2007)

Solution

Given

Hz=Y(z)X(z)=zz-0.97

Hz=Y(z)X(z)=11-0.97z-1

Xz=Yz-0.97z-1Y(z)

Taking inverse z-transform on both sides we get

y(n) - 0.97y(n - 1) = x(n)

y(n) = 0.97y(n - 1) + x(n)

Let y' (n) be the response of the system when the product is quantized by rounding

y' (n) = Q[O.97y' (n - 1)] + x(n)

For a causal filter

y(n) = 0, for n <0

let,

xn=11,  &n=00,  &n≠0

when n=0,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (0) = Q [0.97 Y (-1) +x(O)]

= Q [0.97 x 0] + 11

Y’ (0)  = 11

when n=1,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (1) = Q [0.97 Y (0) +x(1)]

= Q [0.97 x 11] + 0

= Q[10.67]

Covert to binary                      round to 1 decimal                   convert to decimal

10.6710                                        1010.1012                                  1010.12                                    10.510

Y(1) = 10.510

when n=2,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (2) = Q [0.957Y (1) +x(2)]

= Q [0.97 x 10.5] + 0

= Q[10.185]

Covert to binary                      round to 1 decimal                   convert to decimal

10.18510                               1010.0012                                  10102                                              1010

Y(2) = 1010

when n=3,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (3) = Q [0.97 Y (2) +x(3)]

= Q [0.97 x 10] + 0

= Q[9.7]

Covert to binary                      round to 1 decimal                   convert to decimal

9.710                                      1001.1012                                  1001.12                                       9.510

Y(3) = 9.510

when n=4,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (4) = Q [0.97 Y (3) +x(4)]

= Q [0.97 x 9.5] + 0

= Q[9.215]

Covert to binary                      round to 1 decimal                   convert to decimal

9.21510                                      1001.0012                                  10012                                       910

Y(4) = 910

when n=5,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (4) = Q [0.97 Y (4) +x(5)]

= Q [0.97 x 9] + 0

= Q[8.73]

Covert to binary                      round to 1 decimal                   convert to decimal

8.7310                                 1000.1012                                  1000.12                                       8.510

Y(5) = 8.510

when n=6,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (6) = Q [0.97 Y (5) +x(6)]

= Q [0.97 x 8.5] + 0

= Q[8.245]

Covert to binary                      round to 1 decimal                   convert to decimal

8.24510                                      1000.0012                                  10002                                        810

Y(6) = 810

when n=7,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (7) = Q [0.97 Y (6) +x(7)]

= Q [0.97 x 8] + 0

= Q[7.76]

Covert to binary                      round to 1 decimal                   convert to decimal

7.7610                                      0111.1102                                  10002                                        810

Y(7) = 810

Thus, y' (7) = y' (6) and hence for all values of n≥ 6, y' (n) will remain as 8.

Therefore, the system enters into limit cycle when n = 6

Dead band=±2-b1-a=-2-b1-a,2-b1-a

Dead band=±2-21-0.97=-2-21-0.97,2-21-0.97

=[ -8.333 , + 8.333]

Thus, the dead band interval is [-8.333,8.333].

Problem 2

A causal filter is defined by the difference equation

Y(n) =x(n) - 0.9y(n - 1)

The unit sample response hen) is computed such that the computed values are rounded to one decimal place. Show that the filter exhibits dead band effect. Determine the dead band range. (Anna University, May, 2007)

Solution Given

y(n) = x(n) - 0.9y(n - 1)

let,

xn=12,  &n=00,  &n≠0

when n=0,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (0) = -Q [0.9 Y (-1) +x(O)]

= -Q [0.9 x 0] + 12

Y’ (0)  = 12

when n=1,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (1) = -Q [0.9Y (0) +x(1)]

= -Q [0.9 x 12] + 0

Y’ (1)  = -Q[10.8]

Covert to binary                      round to 1 decimal                   convert to decimal

10.810                                        1010.1102                                  10112                                                   1110

Y’ (1)  = -Q[10.8]

Y(1) = -1110

when n=2,

y’(n)=Q[ay’(n-1)+ x(n)]

Y’ (2) = -Q [0.97Y (1) +x(2)]

= -Q [0.9 x -11] + 0

Y’ (2) = -Q[-9.9]

Covert to binary                      round to 1 decimal                (2’s compliment)

-9.910                                          0110.0012                                 01102                                                  10102

(2’s compliment)

convert to decimal

-(10)10

Y’ (2) = -Q[-9.9]

Y(2) = 1010

when n=3,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (3) = -Q [0.9Y (2) +x(3)]

= -Q [0.9 x 10] + 0

Y’ (3) = -Q[9]

Covert to binary                      round to 1 decimal                   convert to decimal

9.10                                      10012                                              10012                                       910

Y’ (3) = -Q[9]

Y(3) = -910

when n=4,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (4) = -Q [0.9Y (3) +x(4)]

= -Q [0.9x -9] + 0

= -Q[-8.1]

Covert to binary                      round to 1 decimal                (2’s compliment)

-8.110                                          1000.0002                                  10002                                            -10002

(2’s compliment)

convert to decimal

-(8)10

Y’ (4) = -Q[-8]

Y(4) = 810

when n=5,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (5) = -Q [0.9 Y (4) +x(5)]

= -Q [0.9x 8] + 0

Y’ (5)  = -Q[7.2]

Covert to binary                      round to 1 decimal                   convert to decimal

7.210                                 0111.0012                                     00012                                        710

Y’ (5)  = -Q[7.2]

Y(5) = -710

when n=6,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (6) = -Q [0.9 Y (5) +x(6)]

= -Q [0.9x -7] + 0

Y’ (6)  = -Q[-6.3]

Covert to binary                      round to 1 decimal                (2’s compliment)

-6.310                                          1001.1102                                  10102                                            -(0110)2

(2’s compliment)

convert to decimal

-(6)10

Y’ (6)  = -Q[-6.3]

Y(6) = -610

when n=7,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (7) = -Q [0.9 Y (6) +x(7)]

= -Q [0.9 x 6] + 0

Y’ (7)  = -Q[5.4]

Covert to binary                      round to 1 decimal                   convert to decimal

5.410                                           0101.0112                                  0101.12                                        5.510

Y’(7)  = -Q[5.4]

Y (7)  = -5.510

when n=8,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (8) = -Q [0.9 Y (7) +x(8)]

= -Q [0.9x -5.5] + 0

Y’ (8)  = -Q[-4.95]

Covert to binary                      round to 1 decimal                (2’s compliment)

-4.9510                                          1011.0012                                  10112                                            -(0101)2

(2’s compliment)

convert to decimal

-(5)10

Y’ (8)  = -Q[-5]

Y(8) = 510

when n=9,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (9) = -Q [0.9 Y (8) +x(9)]

= -Q [0.9 x 5] + 0

Y’ (7)  = -Q[4.5]

Covert to binary                      round to 1 decimal                   convert to decimal

4.510                                           0100.102                                  0100.12                                        4.510

Y’(9)  = -Q[4.5]

Y (9)  = -4.510

when n=10,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (10) = -Q [0.9 Y (9) +x(10)]

= -Q [0.9x -4.5] + 0

Y’ (8)  = -Q[-4.05]

Covert to binary                      round to 1 decimal                (2’s compliment)

-4.0510                                          1100.0002                                  11002                                            -(0100)2

(2’s compliment)

convert to decimal

-(4)10

Y’ (10)  = -Q[-4]

Y(10) = 410

when n=11,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (11) = -Q [0.9 Y (10) +x(11)]

= -Q [0.9 x 4] + 0

Y’ (11)  = -Q[3.6]

Covert to binary                      round to 1 decimal                   convert to decimal

3.610                                           0011.1002                                  0011.12                                        3.510

Y’(11)  = -Q[3.5]

Y (11)  = -3.510

when n=12,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (12) = -Q [0.9 Y (11) +x(12)]

= -Q [0.9x -3.5] + 0

Y’ (12)  = -Q[-3.15]

Covert to binary                      round to 1 decimal                (2’s compliment)

-3.1510                                          1100.1112                                  11012                                            -(0011)2

(2’s compliment)

convert to decimal

-(3)10

Y’ (12)  = -Q[-3]

Y(12) = 310

when n=13,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (13) = -Q [0.9 Y (12) +x(13)]

= -Q [0.9 x 3] + 0

Y’ (13)  = -Q[2.7]

Covert to binary                                  round to 1 decimal                   convert to decimal

2.710                                           0010.1012                                  0010.12                                        2.510

Y’(13)  = -Q[2.5]

Y (13)  = -2.510

Y(13) = -2.510

when n=14,

y’(n)=-Q[ay’(n-1)+ x(n)]

Y’ (14) = -Q [0.9 Y (13) +x(14)]

= -Q [0.9x -2.5] + 0

Y’ (14)  = -Q[-2.25]

Covert to binary                      round to 1 decimal                (2’s compliment)

-2.2510                                          1101.102                                  1101.12                                            -(0010.1)2

(2’s compliment)

convert to decimal

-(2.5)10

Y’ (14)  = -Q[-2.5]

Y(14) = 2.510

Y(14) = 2.510

Thus for all values n≥13 y’(n) remain in the same magnitude as 2.5 with alternative sign. Therefore the system enters into an limit cycle at n=13.

Dead band=±2-b1-a=-2-b1-a,2-b1-a

Dead band=±2-21-0.9=-2-21-0.9,2-21-0.9

Dead band	=[ -2.5 , + 2.5].

Signal Scaling

Saturation arithmetic eliminates limit cycles due to overflow, but it causes undesirable signaldistortion due to the nonlinearity of the clipper. In order to limit the amount of non-linear distortion, it is important to scale the input signal and the unit sample response between the input and any internal summing node in the system such that overflows becomes a rare event.

Let us consider a 2nd order IIR filter as shown in Fig. . A scale factor So is introduced between the input x(n) and the adder 1, to prevent overflow at the output adder 1.

Now the overall input-output transfer function is

From Fig.  we have,

If the instantaneous energy in the output sequence w( n) is less than the finite energy

in the input sequence then, there will not be any overflow.

From above Eq. we have,

Which gives,

dθ=dzjz

Substituting above two equations in parseval’seqn,

Where,

Problem 1

Given

Hz=0.5+0.4z-11-0.312z-1

is the transfer function of a digital filter, find the scaling factor So to avoid overflow in adder 1 of the digital filter shown in Fig.

Problem 1

The input to the system

y(n) = 0.999y(n - 1) + x(n) is applied to an ADC.

What is the power produced by the quantization noise at the output of the filter if theinput is quantized to (a) 8 bits (b) 16 bits. (AU ECE May'07)

Solution

Given             y(n) = 0.999y(n - 1) + x(n).

Taking z-transform on both sides we have

Y(z) = 0.999z-1y(z) + X(z)

Taking Inverse z-transform

h(n) = (0.999)n u(n)

The quantization noise power at the output of the digital filter is

Problem 1

Consider the transfer function where  H(z) = H1(z)H2(z)

Let  H(z) =H1(Z)H2(Z), i.e.,

H1z=11-0.5z-1and H2z=11-0.6z-1

- Find the output roundoff noise power.

The roundoff noise model for H(z) = HI (Z)H2(Z) is shown in Figure .  From the realization, the noise transfer function seen by noise source e 1(n) is written as,

H(Z)=11-0.5z-1 (1-0.6z-1)

- The noise transfer function seen by e2(n) is written as

H2z=11-0.6z-1

- Output noise power due to the noise signal el (n) is,

- σe012= σe22πjHz Hz-1 z-1dz

- Here,

Hz Hz-1 z-1dz=z-11-0.5z-1 1-0.6z-11-0.5z(1-0.6z)

- Here, the stable poles are 0.5 and 0.6 and the unstable pole are (1/0.5) and (1/0.6). Only consider the stable pole.

- ResHzHz-1z-1z=0.5=

- =z-0.5z-1z-0.5(z-0.6) (z-1-0.5) (z-1-0.6)z=0.5

= -9.5238

- ResHzHz-1z-1z=0.6=

- =z-0.6z-1z-0.5(z-0.6) (z-1-0.5) (z-1-0.6)z=0.6

= 13.3928

σe012=σe2[-9.5238+13.3928]

- σe012=σe2[3.8690]

- The steady state output noise power due to e2(n) is,

- σe022= σe22πjH2zH2z-1z-1dz

H2zH2z-1z-1 =  z-1z-0.6(z-1-0.6)

Here the stable poles are 0.6 and the unstable poles are (1/0.6). Only consider the stable pole

ResH2zH2z-1z-1z=0.6=z-0.6z-1(z-0.6) (z-1-0.6)z=0.6

- =1.5626

- σe012= σe22πjHz Hz-1 z-1dz

- σe012=σe2[1.5626]

- Total Output noise power

- σe02=σe01+2σe022

- = σe2[3.8690 + 1.5626 ]

- σe02=σe2[5.4315]

For Example b= 4 bits,
σe2=q212

q=R2b=224=123=18

σe2=(18)212=q212

σe2=1.3021X 10-3

- σe02=σe2[5.4315]

- σe02=1.3021X 10-3   X    [5.4315]

- The total roundoff noise power is given by,

σe02=7.0718 X 10-3

Problem1

Consider the transfer function H(z) = Hl(Z)H2(Z)where

Find the output roundoff noise power.

)viution

The roundoff noise model for H(z) = H1(Z)H2(Z)is given by

From the realization we can find that the noise transfer function seen bynoise

source e1(n) is H(z), where

as,thenoise transfer function seen by e2(n) is

Total steadystate noise variance can be obtained from eqn.

Thus we have

Thus from the equation

We have,

If a1and a2are less than 1 then the poles z = 1/aland z = 1/a2lies outside ofcircleIzl= 1. So, the residues of

H(z)H(z-l)z-lat z = 1/aland z = 1/a2zero.Consequently we have,

- In the same way,

2.Find the study state variance of the noise in the output due to quantization of input for the first order filter(may 11)

Solu:

Y(n)=ay(n-1)+x(n)

Taking Z-transform on both sides. We have

Y(Z)=aZ-1Y(Z)+X(Z)

H(Z)=Y(Z)X(Z)=11-aZ-1=ZZ-a

H(Z-1)=Z-1Z-1-a

WE KNOW

σe2=σe212πjcHZH(Z-1)Z-1dZ

σe2=σe212πjcZ-1(Z-1-a)Z(Z-a)Z-1dZ

=σe2residue of Z-1Z-aZ-1-aZ=a+residue of Z-1Z-aZ-1-aZ=1a

=σe2(Z-a) Z-1Z-aZ-1-aZ=a

=σe2a-1a-1-a

=σe211-a2

=2-2b1211-a2

The impulse response for the above filter is given by h(n)=anu(n)

σe2=σe2n=0∞h2n=σe2n=0∞a2n

=σe21+a2+a4+…+∞                                                              u(n)= 0        for n<0                    1                           for n≥0=σe211-a2=2-2b1211-a2

5) Realize the first order transfer function H(z)=11-aZ-1 and draw its quantization noise model. Find the steady state noise power due to product round off.

σ02=σe212πjcH1 ZH1 (Z-1)Z-1dZ

σ02=σe212πjc11-aZ-111-aZZ-1dZ

=σe2(Z-a)Z-11-aZ-1(1-aZ)Z=a

σ02=2-2b1211-a2

- The output signal of A/D converter is passed through a first order LPF, with transfer function given by

Hz=1-aZZ-a                  for 0<Find the steady state output noise power due to quantization at the output of

digital filter.

HZ-1=1-aZ-1Z-1-a

σ∈2=σe212πjcH1 ZH1 (Z-1)Z-1dZ

σ∈2=σe212πjc1-a2Z-1(Z-1-a)(Z-a)dZ

Pole Z=a, Z=1/a

=σe2z-a1-a2Z-1(Z-1-a)(Z-a)+0

=σe21-a21-a2

- If the system is realized in direct for (may 2011)

H(Z)=11-0.5Z-1(1-0.4Z-1)

= 1(1-0.9Z-1+0.2Z-2)

σ012=σ022

σ012= σe212πjc11-0.5Z-111-0.4Z-111-0.5Z11-0.4ZZ-1dZ

σ012= σe22.38

σ02= 2σ012

=2(2.38σe2)=4.76σe2=4.762-2b12=2-2b(0.3966)

IMPORTANT TWO MARKS

1.What are the effects of finite word length?(May2009)      Pg.no.4

2.What are the types of quantization errors or noise?(May2009& Nov 2010)   Pg.no.4

3.What is input quantization error?(Nov 2009)                           Pg.no.4

4.What is coefficient quantization error or coefficient inaccuracy?(Nov 2011)  Pg.no.4

5.Comparison of fixed point and floating point arithmetic.(Nov 2012 & May 2012) Pg.no.5

6.What is truncation?(May2011)    Pg.no.5

7.What is rounding?(May2012)  Pg.no.5

8.What is overflow limit cycle oscillation?(May2009)  Pg.no.6

9.What is dead band?(May2008)  Pg.no.6

10.What is zero input limit cycle oscillations?(MAY2010)   Pg.no.6

IMPORTANT 16 MARKS

1.For the second order IlR filter, the system function is,

Find  the effect of shift in pole location with 3 bit coefficient representation in direct and cascade forms.(MAY- 2012). Pg.no.15

2.Explain the characteristics of a limit cycle oscillation with respect to the system described by the equation

y(n) = 0.95y(n - 1) + x(n).

Determine the dead band of the filter.(Assume sign magnitude is 5 bit).(May-2012)Pg.no.22

3.An IIR causal filter has the system function

Assume that the input signal is zero valued and the computed output signal values are rounded to one decimal place. Show that under those stated conditions, the filter output exhibits dead band effect. What is the dead band range? (Anna University, May, 2007) Pg.no.24

4.A causal filter is defined by the difference equation

Y(n) =x(n) - 0.9y(n - 1)

The unit sample response hen) is computed such that the computed values are rounded to one decimal place. Show that the filter exhibits dead band effect. Determine the dead band range. (Anna University, May, 2007)Pg.no.26.

5.The output signal of A/D converter is passed through a first order LPF, with transfer function given by

Hz=1-aZZ-a<Find the steady state output noise power due to quantization at the output of

digital filter. (May-2011)Pg.no.40

6) Realize the first order transfer function H(z)=11-aZ-1 and draw its quantization noise model. Find the steady state noise power due to product round off. (May 2011)Pg.no.39